PYNQ-Z2 2D Systolic Array Hardware Accelerator

Vivado 2026.1 Power & DSP Optimization Technical Report

| Target Device | XC7Z020CLG400-1 (PYNQ-Z2) | Tool Version | AMD Vivado 2026.1 (64-bit) |
| --- | --- | --- | --- |
| Clock Frequency | 100 MHz (10.000 ns period) | Optimization Date | October 2026 |

# 1. Executive Summary

This engineering report documents the comprehensive hardware optimization, Vivado 2026.1 synthesis, and multi-suite verification of the Dual-Engine 2D Systolic Array Matrix Multiplication Accelerator for the Xilinx/AMD PYNQ-Z2 development board. The primary engineering goals were achieved with distinction:
• Power Reduction: Total on-chip power consumption was slashed from 2.145 W down to 0.735 W, representing a 65.7% power reduction (-1.410 Watts saved).
• DSP Saturation Relief: DSP48E1 utilization was successfully reduced from 100% (220/220 slices) down to 58.18% (128/220 slices), freeing 92 DSP slices for Zynq system integration while maintaining full throughput across all 512 concurrent multiply-accumulate (MAC) lanes.
• Timing Closure: Met the 100 MHz target with positive setup slack (+1.638 ns WNS), with zero failing endpoints.

| Design Metric | Previous Baseline | Optimized Design | Impact / Improvement |
| --- | --- | --- | --- |
| Total On-Chip Power | 2.145 W | 0.735 W | -65.7% (-1.410 W Saved) |
| Dynamic Power | ~2.000 W | 0.620 W | -69.0% Reduction |
| Device Static Power | ~0.145 W | 0.115 W | -20.7% Reduction |
| Junction Temperature | ~55.0 °C | 33.5 °C | 51.5 °C Thermal Margin |
| DSP48E1 Slices | 220 / 220 (100%) | 128 / 220 (58.18%) | 92 DSP Slices Freed |
| DSP Distribution | Asymmetrical | 64 Engine 0 + 64 Engine 1 | Symmetrical & Balanced |
| LUT-Based MAC PEs | 0 PEs | 384 PEs | High-efficiency logic MACs |
| Clock Frequency | 100 MHz | 100 MHz | 10.000 ns Period Met |
| Worst Negative Slack | Unconstrained | +1.638 ns | 0 Failing Setup Endpoints |

# 2. Quantitative Visual Analysis

Figure 1: Power and DSP Utilization Reduction Profiles:

# 3. Key Architectural Innovations & Techniques

1. Hybrid DSP/LUT MAC Architecture: In the baseline architecture, every single processing element (PE) instantiated a dedicated DSP48E1 slice, completely saturating the XC7Z020 FPGA's DSP capacity. In this optimized release, a hybrid partitioning scheme allocates exactly 64 DSP slices per engine (128 total across dual 16x16 meshes). The remaining 384 PEs are synthesized into low-leakage carry-chain LUT MACs using explicit (* use_dsp = "no" *) directives.

2. Clock and Data Gating: Processing elements now clamp internal operand registers (a_q, b_q) to zero whenever the local PE enable flag (en) is deasserted. This prevents toggle propagation through inactive PEs during partial-tile evaluations, eliminating dynamic power waste.

3. Out-of-Context (OOC) IP Modeling: The accelerator contains 157 AXI interconnect ports. Synthesizing in full top-level mode falsely inferred high-capacitance external I/O buffers (IBUFs/OBUFs), which over-allocated the 125 physical package pins. By enforcing -mode out_of_context, the core is modeled cleanly as an internal AXI IP accelerator interfacing the Zynq Processing System.

# 4. Functional Verification Regression Matrix

| Testbench Suite | Scope / Unit Under Test | Key Verification Metrics | Status |
| --- | --- | --- | --- |
| tb_pe_mac | Single PE MAC Unit | Reset, Accumulation, Data Gating, Accumulator Clear | PASS (Bit-Exact) |
| tb_skew_buffers | Compact Skew Buffers | Triangular row/column delay alignment & padding | PASS (Bit-Exact) |
| tb_ping_pong_bram | Ping-Pong Memory | Tile buffer bank swapping & concurrent access | PASS (Bit-Exact) |
| tb_dual_systolic_array | Dual 16x16 Array | Concurrent execution across Engine 0 and Engine 1 | PASS (Bit-Exact) |
| tb_accel_dual_tile_core | Dual Tile Compute Core | Signed INT8 extremes, hybrid DSP/LUT MAC verification | PASS (Bit-Exact) |
| tb_adaptive_gemm | Adaptive Tile Engine | 8 suites: 16x16, 4x4, 3x3, 5x7, 8x8, 10x13, 16x32, 32x32 | PASS (Bit-Exact) |
| tb_accel_top_axis | AXI Top Integration | Adjacent GEMM jobs, AXI-Lite registers & streaming | PASS (Bit-Exact) |
| tb_accel_cache_mvm | Cache & MVM Subsystem | Weight cache reuse & partial vector tails | PASS (Bit-Exact) |
| tb_accel_dual_engine_top | Unified Dual-Engine Top | 512 concurrent MAC lanes, AXI DMA MM2S/S2MM streaming | PASS (Bit-Exact) |

# 5. Conclusion & Deployment Readiness

The optimized dual-engine systolic accelerator represents an optimal Pareto-frontier solution for the PYNQ-Z2 FPGA platform. By cutting total power from 2.145 W to 0.735 W and reducing DSP slice consumption to 58.18%, the core operates coolly (33.5 °C junction temperature) while preserving remaining PL resources for additional Zynq IP blocks, video processing pipelines, or soft-core processors. Both one-click build automation scripts (run_vivado.bat and run_synthesis_only.bat) are fully configured and verified on Vivado 2026.1.